requirement

Author(s):		jacky

Published on:	10-Jun-2024

Table of Contents

Chapter 1: Requirement Set: requirement	6

1 5.1 Input /  输入（网络开关）	7

2 5.2 Input /  输入（硬线开关）	7

2.1 5.2.1 Front Wiper Switch Request/前雨刮开关请求	7

2.2 5.2.2 Front Wiper Sensitive Switch Request/前雨刮灵敏度调节开关请求	8

2.3 5.2.3 Front Wiper Park Position Input /前雨刮停止位输入	8

2.4 5.2.4 Rear Wiper Park Position Input /后雨刮停止位输入	9

2.5 5.2.5 Priority of Front Wiper Inputs/前雨刮输入优先级	9

3 5.3 Signal description /信号描述说明	9

4 5.4 Feedback of Front Wiper Output Status/前雨刮输出状态反馈	10

5 5.5 Front Wiper Low Speed Mode/前雨刮低速模式	10

5.1 5.5.1 Activation of Front Wiper Low Speed Mode/开启前雨刮低速模式	11

5.2 5.5.2 Deactivation of Front Wiper Low Speed Mode/退出前雨刮低速模式	11

5.3 5.5.3 Deactivation Sequence of Front Wiper Low Speed/前雨刮低速退出时回位	12

5.4 5.5.4 Change to INT Mode /低速模式转移至间歇模式	13

5.5 5.5.5 Change to AUTO Mode /低速模式转移至自动模式	13

5.6 5.5.6 Change to HIGH Mode /低速模式转移至高速模式	14

6 5.6 Front Wiper High Speed Mode/前雨刮高速模式	14

6.1 5.6.1 Activation of Front Wiper High Speed Mode/开启前雨刮高速模式	14

6.2 5.6.2 Deactivation of Front Wiper High Speed Mode /退出前雨刮高速模式	15

6.3 5.6.3 Deactivation Sequence of Front Wiper High Speed Mode/前雨刮高速退出时回位	15

6.4 5.6.4 Change to INT Mode/高速模式转移至间歇模式	16

6.5 5.6.5 Change to AUTO Mode/高速模式转移至自动模式	17

6.6 5.6.6 Change to LOW Mode/转移至低速模式	17

7 5.7 Front Wiper Intermittent Mode/前雨刮间歇模式	18

7.1 5.7.1 Intermittent Mode with RLS Option Disabled/雨量传感器配置间歇模式	18

7.2 5.7.2 Activation of Front Wiper Intermittent Mode/前雨刮间歇模式开启	19

7.3 5.7.3 Activation Sequence of Front Wiper Intermittent Mode/前雨刮间歇模式工作方式	19

7.4 5.7.4 Deactivation of Front Wiper Intermittent Mode/退出前雨刮间歇模式	20

7.5 5.7.5 Change to LOW Mode/前雨刮间歇模式转移至低速模式	21

7.6 5.7.6 Change High Mode/前雨刮间歇模式转移至高速模式	21

7.7 5.7.7 Dwell Time with Rheostat and Vehicle Speed/间歇时间	22

7.8 5.7.8 Default Position If Rheostat Switch Failure/间歇档位开关故障默认值	22

7.9 5.7.9 Default Speed Class If Vehicle Speed Signal Failure /车速信号故障默认车速档位	23

7.10 5.7.10 Dwell Time Change If Vehicle Speed Class Changing/车速变化导致间歇时间变 化	23

7.11 5.7.11 Reduction of Dwell Time Changed by Rheostat Position/间歇档位降低间歇时 间	24

7.12 5.7.12 Extension of Dwell Time Changed by Rheostat Position/间歇档位增加间歇时间	25

7.13 5.7.13 Hysteresis for Vehicle Speed Class Change/车速档位变化滞回	25

8 5.8 Front Wiper Auto Mode/前雨刮自动模式	26

8.1 5.8.1 Auto Mode with RLS Option Enabled/雨量传感器使能自动模式	26

8.2 5.8.2 Front Wiper Control via RLS Request Signal/雨量传感器控制前雨刮	27

8.3 5.8.3 Transition between Different Modes/不同模式转移	27

8.4 5.8.4 Front Wiper Control via RLS Splash Request/雨量传感器溅水命令	28

8.5 5.8.5 RLS Sensitivity Selection/雨量传感器灵敏度调节	29

8.6 5.8.6 Increase the RLS Sensitivity by Rheostat Change/档位开关调整灵敏度增加	29

8.7 5.8.7 Fail Safe Strategy If RLS Fault or Communication Error/故障处理策略	29

8.8 5.8.8 Change to LOW Mode/自动模式转移至低速模式	30

8.9 5.8.9 Change to HIGH Mode/自动模式转移至高速速模式	31

8.10 5.8.10 洗车模式下禁止自动雨刮功能	31

9 5.9 Front Wiper Mist Mode/前雨刮 Mist 操作	32

10 5.10 Front Washer and Wash Wiping /前洗涤及雨刮洗涤联动	32

10.1 5.10.1 Activation of Front Washer/前洗涤开启	32

10.2 5.10.2 Deactivation of Front Washer/关闭前洗涤	33

10.3 5.10.3 Not Activation of Front Wash Wiping/前雨刮洗涤联动不开启	33

10.4 5.10.4 Activation of Front Wash Wiping during INT Mode or OFF Mode/OFF 或 INT 模 式激活雨刮洗涤联动	34

10.5 5.10.5 Activation of Front Wash Wiping during LOW or HIGH Mode/LOW 或 HIGH 模 式激活雨刮洗涤联动	34

10.6 5.10.6 Activation of Front Washer during Auto Mode with RLS Request is Low Speed  or High Speed Mode or Splash Mode/自动模式下 RLS 请求为低速或高速或 Splash 时 激活雨刮洗涤联动	35

10.7 5.10.7 Activation of Front Wash Wiping during Auto Mode with RLS Request is OFF/ 自动模式下 RLS 请求为 OFF 时激活雨刮洗涤联动	36

10.8 5.10.8 Front Washer Prolongation Wiping/雨刮联动延续刮擦	36

10.9 5.10.9 Change the Front Wiper Switch during Wash Wiping or Prolongation Wiping/ 洗涤联动或联动延续刮擦过程中前雨刮开关变化	37

10.10 5.10.10 Ignition Changed to OFF during Wash Wiping/雨刮洗涤联动过程电源模式变为 OFF	38

10.11 5.10.11 Detection of a Faulty Front Washer Switch/前洗涤开关故障检测	38

10.12 5.10.12 Behavior after a Faulty Front Washer Switch/前洗涤开关故障处理	39

10.13 5.10.13 Feedback of Front Washer Output Status/前洗涤输出状态反馈	39

11 5.11 Front Wiper Park Position/前雨刮回位位置	40

11.1 5.11.1 Reaction Time to the Detection of Park Switch Close/回位信号响应时间	40

11.2 5.11.2 Back to Park Position When Ignition switched ON/电源模式拨至 ON 时回位	41

12 5.12 Front Wiper Stall Protection/前雨刮堵转保护	41

12.1 5.12.1 Deactivate Front Wiper after Maximum Wait Time for Wiper Reaching Park  Position/最大等待雨刮回位时间	41

12.2 5.12.2 Reactivate Front Wiper after Stall Protection/前雨刮堵转保护后再次开启	42

13 5.13 Front Wiper Maintenance Mode/前雨刮维修模式	43

13.1 5.13.1 网络输入进入前雨刮维修模式	43

13.1.1 5.13.1.1 开启前雨刮维修模式条件	43

13.1.2 5.13.1.2 Action of Front Wiper Maintenance Mode/维修模式下前雨刮动作	44

13.1.3 5.13.1.3 Quit The Front Wiper Maintenance Mode/退出前雨刮维修模式	44

13.2 5.13.2 开关输入进入前雨刮维修模式	45

13.2.1 5.13.2.1 开启前雨刮维修模式条件	45

13.2.2 5.13.2.2 定时器关闭	46

13.2.3 5.13.2.3 Action of Front Wiper Maintenance Mode/维修模式下前雨刮动作	46

13.2.4 5.13.2.4 Quit The Front Wiper Maintenance Mode/退出前雨刮维修模式	47

13.3 5.13.3 Open The Hood When Under The Front Wiper Maintenance Mode/维修模式下 前舱盖打开	48

13.4 5.13.4 前雨刮维修模式存储	48

13.5 5.13.5 前雨刮维修模式下电源模式变为 OFF 后雨刮动作	48

14 5.14 Rear Wiper Switch Request/后雨刮开关请求（预留）	49

15 5.15 Rear Washer and Wash Wiping/后洗涤及雨刮洗涤联动（预留）	49

16 5.16 Reverse Gear Wiping/倒车雨刮联动（预留）	50

17 5.17 Rear Wiper and Washer Inhibit/后雨刮及洗涤禁止（预留）	50

18 5.18 Rear Wiper Park Position/后雨刮回位位置（预留）	50

19 5.19 Rear Wiper Stall Protection/后雨刮堵转保护（预留）	51

20 5.20 Voice Control of Front Wiper in Low Speed Mode/语音控制前雨刮低速运行	51

20.1 5.20.1 Activation of Front Washer Voice Control/前雨刮语音控制功能开启	52

20.1.1 5.20.1.5 Activation of Front Wiper Voice Control/前雨刮语音控制功能开启条件	52

20.1.2 5.20.1.6 Output When Activating Front Wiper Voice Control/前雨刮语音控制功能开启动作	53

20.2 5.20.2 Deactivation of Front Wiper Voice Control/前雨刮语音控制功能退出	53

20.2.1 5.20.2.1 Deactivation of Front Wiper Voice Control/前雨刮语音控制退出条件	53

20.2.2 5.20.2.2 Deactivation of Front Wiper Voice Control In the case of iginition status change /当电源模 式变化时退出前雨刮语音控制	54

20.2.3 5.20.2.3   Deactivation of Front Wiper Voice Control In the Case of Front Washer Switch Change 前 洗涤开关有效退出前雨刮语音控制	54

20.2.4 5.20.2.4   Deactivation of Front Wiper Voice Control In the Case of Front Wiper Switch Change    /前 雨刮开关由 OFF 拨到非 OFF 档位退出前雨刮控制。	55

20.2.5 5.20.2.5 Deactivation  of  Front  Wiper  Voice  Control  In  the  Case  of  Receiving  Voice  Control  of  Deactivating Front Wiper/收到语音控制命令退出前雨刮语音控制	55

20.3 5.20.3 Conflicts Management during of Voice Control Front Wiper/冲突处理	56

21 5.21 Voice Control of Front Washer/前洗涤及雨刮洗涤联动语音控制	56

21.1 5.21.1 Activation of Front Washer Voice Control/前洗涤语音控制功能开启	57

21.1.1 5.21.1.1 Activation of Front Washer Voice Control/前洗涤语音控制功能开启条件	57

21.1.2 5.21.1.2 Front Wash Output when Activating Front Washer Voice Control /前洗涤语音控制开启动作	58

21.1.3 5.21.1.3 Front Wash Wiping when Activating Front Washer Voice Control /前洗涤语音控制雨刮联动	58

21.1.4 5.21.1.4 Prolongation Wiping when Activating Front Washer Voice Control /前洗涤语音控制延续刮擦	59

21.2 5.21.2 Deactivation of Front Washer Voice Control/前洗涤语音控制退出	59

21.2.1 5.21.2.5 Deactivation of Front Washer Voice Control/前洗涤语音控制退出条件	60

21.2.2 5.21.2.6 Deactivation of Front Washer Voice Control In the case of iginition status change /当电源 模式变化时退出前洗涤语音控制	60

21.2.3 5.21.2.7   Deactivation of  Front Washer Voice Control  In  the Case of Front Wash Switch Status  Change 前洗涤开关有效退出前洗涤语音控制	61

21.2.4 5.21.2.8   Deactivation of Front Washer Voice Control In the Case of Front Wash Status Change/前 雨刮开关由 OFF 拨到非 OFF 档位退出前洗涤语音控制。	61

21.2.5 5.21.2.9 Deactivation of Front Washer Voice Control In the Case of Receiving Voice Control of Front  Wiper/收到前雨刮语音控制命令退出前洗涤语音控制	62

21.3 5.21.3 Conflicts Management during of Voice Control Front Wash/冲突处理	62

21.4 5.21.4 Fault Management/故障处理	63

22 5.22 Front Wiper at Auto Drive Mode/自动驾驶模式下的自动前雨刮功能（预留）	63

23 5.23 雨刮开关信号发送（预留）	63

24 5.24 雨量光需求固定信号发送说明	64

25 5.25 雨量光需求检测信号发送说明	64

26	65

26.1 no need to do debounce by the model	65

26.2 input and output signal description only	65

26.3 Reserved	65

26.4 no software related design	66

Chapter 1: Requirement Set: requirement

Description

Implementation Status

Total: 103, Implemented: 67, Justified: 13, None: 23

Verification Status

Total: 103, Passed: 27, Justified: 13, Failed: 0, Unexecuted: 0, None: 63

1 5.1 Input /  输入（网络开关）

Requirement Type    Functional

ID    R_Wiper_Wash_5.1

Description

当 CFG_Wiper&Wash_SW_HardWire_Typ== disabled; 雨刮与洗涤的开启与关闭信号来自网络；

当 CIM_FWiperWashReq= 0x2: Request 时；满足前洗涤的功能输入要求；

当 CIM_FWiperMistReq= 0x2: Request 时；满足前雨刮点刮的输入要求；

当 CIM_FWiperSwtReq=0x2: Auto，雨刮开关在自动档位；CIM_FWiperSwtReq=0x3: Low Speed，雨

刮开关在低速档位；CIM_FWiperSwtReq=0x4: High Speed；

当 CIM_FWiperSwtFault= 0x1: FAULT，此时雨刮开关故障；

备注：BDC 收到相应的网络开关信号，驱动输出与硬线开关检测输出相同；

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

2 5.2 Input /  输入（硬线开关）

Requirement Type    Functional

ID    R_Wiper_Wash_5.2

Description

当 CFG_Wiper&Wash_SW_HardWire_Typ== enabled;车身域控制器开关采集为硬线采集；

Implementation Status

Total: 5, Implemented: 3, Justified: 2, None: 0

Verification Status

Total: 5, Passed: 3, Justified: 2, Failed: 0, Unexecuted: 0, None: 0

2.1 5.2.1 Front Wiper Switch Request/前雨刮开关请求

Requirement Type    Functional

ID    R_Wiper_Wash_5.2.1

Description

When the front wiper column switch on the Intermittent/Auto position, the front wiper working mode is Intermittent speed if the rain sensor function is disabled. And the front wiper working mode is depending on the rain sensor requests if the rain sensor function is enabled, which could be off, low speed or high speed.

当前雨刮开关处于 INT/AUTO 位置，当配置为 CFG_RAIN_LIGHT_SENSOR_ENABLED==disabled，则前雨刮工作在间歇模式；当配置为 CFG_RAIN_LIGHT_SENSOR_ENABLED== enabled，则按照雨量传感器的请求工作在off，低速或高速。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

2.2 5.2.2 Front Wiper Sensitive Switch Request/前雨刮灵敏度调节开关请求

Requirement Type    Functional

ID    R_Wiper_Wash_5.2.2

Description

灵敏度开关为硬线时，当配置为CFG_RAIN_LIGHT_SENSOR_ENABLED== enabled，发送 LIN 信号分别为 Level0~Level4 至雨量传感器；当配置为 CFG_RAIN_LIGHT_SENSOR_ENABLED==disabled，为间歇档灵敏度。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

2.3 5.2.3 Front Wiper Park Position Input /前雨刮停止位输入

Requirement Type    Functional

ID    R_Wiper_Wash_5.2.3

Description

滤波时间：18ms

采样频率：6ms/次

采样次数：大于 3 次，中间任意一次采样无效，则计数器重新开始计数。

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

2.4 5.2.4 Rear Wiper Park Position Input /后雨刮停止位输入

Requirement Type    Functional

ID    R_Wiper_Wash_5.2.4

Description

滤波时间：18ms

采样频率：6ms/次

采样次数：大于 3 次，中间任意一次采样无效，则计数器重新开始计数。

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

2.5 5.2.5 Priority of Front Wiper Inputs/前雨刮输入优先级

Requirement Type    Informational

ID    R_Wiper_Wash_5.2.5

Description

参照后续优先级章节

3 5.3 Signal description /信号描述说明

Requirement Type    Functional

ID    R_Wiper_Wash_5.3

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

4 5.4 Feedback of Front Wiper Output Status/前雨刮输出状态反馈

Requirement Type    Functional

ID    R_Wiper_Wash_5.4

Description

前雨刮输出状态需反馈到 CAN 总线上。

当前雨刮输出关闭时，设置 CAN 信号 FrnWiprOutpSts= Off;

当前雨刮低速运行时，设置 CAN 信号 FrnWiprOutpSts= Low;

当前雨刮高速运行时，设置 CAN 信号 FrnWiprOutpSts= High;

注：当雨刮间歇或自动运行时，前雨刮输出状态按照实际雨刮输出状态反馈。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

5 5.5 Front Wiper Low Speed Mode/前雨刮低速模式

Requirement Type    Container

ID    R_Wiper_Wash_5.5

Description

Implementation Status

Total: 6, Implemented: 6, Justified: 0, None: 0

Verification Status

Total: 6, Passed: 6, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

5.1 5.5.1 Activation of Front Wiper Low Speed Mode/开启前雨刮低速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.5.1

Description

On receipt of a valid front wiper LOW speed request, shall run the front wiper at low speed by setting

WP_Front_Wiper_Enable_CMD ON. While the wiper LOW speed request is valid, the wiper shall continue to run at low speed.

当接收到前雨刮低速工作请求时，应当将 WP_Front_Wiper_Enable_CMD 打开驱动雨刮低速运行。当前雨刮低速请求一直有效，前雨刮一直以低速运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

5.2 5.5.2 Deactivation of Front Wiper Low Speed Mode/退出前雨刮低速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.5.2

Description

If the front wiper switch is changed to the OFF mode during low speed wiping, then the wipers shall

continue at low speed wiping until reach the park position.

If Power mode is changed to OFF or ACC or CRANK during low speed wiping, then the wipers shall

be stopped immediately.

如果在前雨刮处于低速刮擦时，开关请求变为 OFF 模式，则前雨刮运动至 Park 位置时停止。

如果在前雨刮处于低速刮擦时，电源模式变为 OFF 档，则前雨刮立即停止。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

5.3 5.5.3 Deactivation Sequence of Front Wiper Low Speed/前雨刮低速退出时回位

Requirement Type    Functional

ID    R_Wiper_Wash_5.5.3

Description

As the front wiper motor is low current version, Should keep the wiper working until front wiper reach

to the park position by detection of wiper park switch WP_Front_Wiper_Park_Sts_SW is active, and then set WP_Front_Wiper_Enable_CMD off.

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

5.4 5.5.4 Change to INT Mode /低速模式转移至间歇模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.5.4

Description

If the front wiper switch is changed to the intermittent mode during low speed wiping cycle, then the

wipers shall continue at low speed until they reach to the park position. The next wipe cycle shall then be initiated following completion of the relevant delay period.

如果在低速刮擦时，前雨刮开关切换至 INT 模式，则控制雨刮低速运行至回位位置。从下刮擦操作之

后开始按照相应的延时运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

5.5 5.5.5 Change to AUTO Mode /低速模式转移至自动模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.5.5

Description

If the front wiper switch is changed to the Auto mode during low speed wiping cycle, then the wipers shall continue at low speed until they reach to the park position. Shall activate the wipers according the rain sensor request signal.

如果在低速刮擦过程中，前雨刮切换至 AUTO 模式，则前雨刮继续运行至回位位置。之后，按照雨量

传感器请求信号控制前雨刮工作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

5.6 5.5.6 Change to HIGH Mode /低速模式转移至高速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.5.6

Description

If the front wiper switch is changed to the high speed mode during low speed wiping cycle, then the

wipers shall immediately operate at high speed.

如果在低速刮擦过程中，前雨刮开关切换至 HIGH 模式，则前雨刮立即切换至高速模式运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

6 5.6 Front Wiper High Speed Mode/前雨刮高速模式

Requirement Type    Container

ID    R_Wiper_Wash_5.6

Description

Implementation Status

Total: 6, Implemented: 6, Justified: 0, None: 0

Verification Status

Total: 6, Passed: 6, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

6.1 5.6.1 Activation of Front Wiper High Speed Mode/开启前雨刮高速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.6.1

Description

On receipt of a valid front wiper high speed request, shall run the front wiper at high speed by setting

both WP_Front_Wiper_Enable_CMD and WP_Front_Wiper_Speed_CMD ON. While the wiper high speed

request is valid the wiper shall continue to run at high speed.

当接收到有效的前雨刮 HIGH 模式请求，应该同时打开输出 WP_Front_Wiper_Enable_CMD 和

WP_Front_Wiper_Speed_CMD 启动前雨刮电机高速运行。当前雨刮高速模式请求一直有效，则前雨刮一

直保持高速运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

6.2 5.6.2 Deactivation of Front Wiper High Speed Mode /退出前雨刮高速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.6.2

Description

If the front wiper switch is changed to the off mode during high speed wiping cycle, then the wipers

shall switch to low speed wiping immediately until reaching to the park position.

If Power mode is changed to OFF or ACC or CRANK during high speed wiping cycle, then the wipers

shall be stopped immediately.

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

6.3 5.6.3 Deactivation Sequence of Front Wiper High Speed Mode/前雨刮高速退出时回位

Requirement Type    Functional

ID    R_Wiper_Wash_5.6.3

Description

As the front wiper motor is low current version, should keep the wiper working at low speed until front

wiper reach to the park position. Shall first set WP_Front_Wiper_Speed_CMD OFF to let the front wiper

work at low speed. Then until detection of wiper park switch WP_Front_Wiper_Park_Sts_SW is active,

shall set WP_Front_Wiper_Enable_CMD to OFF. Details refer to Figure 2.

保持低速运行直至检测到前雨刮运行到 PARK 位置后停止输出。

首 先 切 断 WP_Front_Wiper_Speed_CMD 输 出 使 其 运 行 在 低 速 刮 刷 模 式 ， 然 后 检 测 到

WP_Front_Wiper_Park_Sts_SW 有效时，切断 WP_Front_Wiper_Enable_CMD 输出。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

6.4 5.6.4 Change to INT Mode/高速模式转移至间歇模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.6.4

Description

If the front wiper switch is changed to the INT mode during high speed wiping cycle, then the wipers

shall switch to low speed mode immediately until reaching to the park position. The next wipe cycle shall then be initiated following completion of the relevant delay period.

如果前雨刮在高速刮擦过程中开启切换至间歇模式，则前雨刮电机立即切换至低速运行直到回位位置。

相关的延时在下一刮擦周期执行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

6.5 5.6.5 Change to AUTO Mode/高速模式转移至自动模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.6.5

Description

If the front wiper switch is changed to the AUTO mode during high speed wiping cycle, then the wipers shall switch to low speed mode immediately until reaching to the park position. Shall activate the wipers according the rain sensor request signal.

如果前雨刮在高速刮擦过程中开启切换至自动模式，则前雨刮电机立即切换至低速运行直到回位位置。

在下一个刮擦周期按照雨量传感器的命令控制前雨刮工作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

6.6 5.6.6 Change to LOW Mode/转移至低速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.6.6

Description

If the front wiper switch is changed to the low speed during high speed wiping cycle, then the wipers

shall immediately switch to low speed.

如果前雨刮在高速刮擦过程中开启切换至低速模式，则前雨刮电机立即切换至低速运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7 5.7 Front Wiper Intermittent Mode/前雨刮间歇模式

Requirement Type    Container

ID    R_Wiper_Wash_5.7

Description

Implementation Status

Total: 13, Implemented: 13, Justified: 0, None: 0

Verification Status

Total: 13, Passed: 10, Justified: 0, Failed: 0, Unexecuted: 0, None: 3

7.1 5.7.1 Intermittent Mode with RLS Option Disabled/雨量传感器配置间歇模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.1

Description

If the EEPROM parameter CFG_RAIN_LIGHT_SENSOR_ENABLED is set to false, Shall operate with intermittent mode when the front wiper intermittent/auto switch is selected.

如果 EEPROM 的参数配置为禁止，则当前雨刮 INT/AUTO 开关有效时，控制前雨刮工作在间歇模式。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.2 5.7.2 Activation of Front Wiper Intermittent Mode/前雨刮间歇模式开启

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.2

Description

On receipt of a valid front wiper intermittent mode request, shall run the front wiper at low speed.

When the wipers reach the park position, the front wiper motor is deactivated until the dwell time is expired

and then activated again at low speed. The dwell time is defined by the interval adjust switch and vehicle

speed.

当接收到前雨刮间歇模式工作请求时，应当将 WP_Front_Wiper_Enable_CMD 打开驱动雨刮低速运行。当前雨刮运动

到回位位置时，电机停止工作等待间歇设定时间然后再次打开低速运行。间歇设定时间由间歇档位开关及车速共同决定。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.3 5.7.3 Activation Sequence of Front Wiper Intermittent Mode/前雨刮间歇模式工作方式

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.3

Description

The BCM starts the intermittent wiping by setting WP_Front_Wiper_Enable_CMD ON continuously

until wiper reach to park position. Then set WP_Front_Wiper_Enable_CMD OFF to wait dwell time to start

next wipe cycle. The dwell time is started when the wiper reaches the park position.

处于间歇刮水模式时，将 WP_Front_Wiper_Enable_CMD 输出打开直至前雨刮再次运行至回位位置。然后将WP_Front_Wiper_Enable_CMD 关闭，等待间歇设定时间后开始下一间歇工作周期。间歇设定时间计时开始于前雨刮到达回位位置处。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.4 5.7.4 Deactivation of Front Wiper Intermittent Mode/退出前雨刮间歇模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.4

Description

If the front wiper switch is changed to the off mode during the intermittent wipe cycle; then the wipers

shall continue at low speed until reach the park position.

If Power mode is changed to OFF or ACC or CRANK during the intermittent wipe cycle; then the

wipers shall be stopped immediately.

当前雨刮开关从 INT 模式变为 OFF 模式时，前雨刮继续在低速运行至回位位置。

当前雨刮处于 INT 模式工作时，电源模式变为 OFF 档时前雨刮立即停止工作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.5 5.7.5 Change to LOW Mode/前雨刮间歇模式转移至低速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.5

Description

If the front wiper switch is changed to the low speed mode during intermittent wiping cycle, then the

wipers shall start at low speed mode immediately. If front wiper is in the park position, then it shall be

activated immediately.

如果前雨刮开关请求从 INT 变为 LOW 模式，前雨刮立即进入低速模式工作。如果前雨刮处于回位位置，则立即开始低

速工作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.6 5.7.6 Change High Mode/前雨刮间歇模式转移至高速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.6

Description

If the front wiper switch is changed to the high speed mode during intermittent wiping cycle, then the wipers shall start at high speed mode immediately. If front wiper is in the park position, then it shall be activated immediately.

如果前雨刮开关请求从 INT 到 HIGH 模式，前雨刮立即进入高速模式工作。如果前雨刮处于回位位置，则立即开始高速工作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.7 5.7.7 Dwell Time with Rheostat and Vehicle Speed/间歇时间

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.7

Description

雨刮间歇时间调节为可配置项。

配置参数 CFG_FRONT_WIPER_INT = 0 不具备雨刮间歇时间调节档位

配置参数 CFG_FRONT_WIPER_INT = 1

（默认值）

具备雨刮间歇时间调节档位。

The following table defines the dwell time in second between wipes for the different Rheostat positions with respect to various vehicle speed class.

Note: The maximum tolerance of the dwell time in the above table is 10%.

备注：当配置参数为 CFG_FRONT_WIPER_INT = 0 时，间歇时间按照表格中的 Default 执行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.8 5.7.8 Default Position If Rheostat Switch Failure/间歇档位开关故障默认值

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.8

Description

➢ The values of default position in 错误!未找到引用源。 shall be selected if the BCM detects a fault

condition of the Rheostat switch input.

➢ 如果间歇档位开关输入故障，则使用表格中的默认值。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.9 5.7.9 Default Speed Class If Vehicle Speed Signal Failure /车速信号故障默认车速档位

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.9

Description

The values of speed class 1 (8≤V1<32) in Table 2.2 shall be selected if detects a fault condition for the vehicle speed signal, like vehicle speed is invalid or lost communication with vehicle speed CAN message.

If no vehicle speed signal received after Ignition turned ON, the values of speed class 1 (8≤V1<32) shall be selected too before the valid vehicle speed received.

如果检测到车速信号故障，如 VEHSPDVLD ！= Value OK 或 VEHSPD 报文丢失，则默认使用表格 2.2 中车速 V1 档

（8≤V1<32）。

在电源模式拨至 ON 档且尚未收到车速信号时，也采用默认的车速档位 V1 档（8≤V1<32）。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

7.10 5.7.10 Dwell Time Change If Vehicle Speed Class Changing/车速变化导致间歇时间变 化

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.10

Description

The dwell time should only be continuously checked and changed from one vehicle speed class to another vehicle speed class at a time between wipes. The wait time shall be continuously compared with the target dwell time. If the wait time is longer than the target dwell time, shall activate the wipe immediately.

仅在两次刮擦动作之间的等待过程中，间歇设定时间会根据车速档位变化进行更新。等待时间总是同更新的间歇设定时间进行比较。一旦等待时间长于间歇设定时间，立即开始刮擦。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

7.11 5.7.11 Reduction of Dwell Time Changed by Rheostat Position/间歇档位降低间歇时 间

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.11

Description

When a shorter dwell time is selected due to Rheostat position is changed by user and the wiper is

not moving, an immediate wipe will be performed and then the new dwell time will be used. If the wiper is

moving at the time the dwell time is changed, the new dwell time will be used when the wiper reaches to

the park position.

当用户操作间歇档位开关设定更短的间歇时间时，如果雨刮处于停止状态，则立即刮擦一次，然后使用

新的间歇设定时间；如果间歇设定时间变化时雨刮处于运动状态，则当雨刮运动至回位位置时，使用新的间

歇设定时间。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

7.12 5.7.12 Extension of Dwell Time Changed by Rheostat Position/间歇档位增加间歇时间

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.12

Description

When a longer interval is selected due to Rheostat position is changed by user and the wiper is not

moving, the duration of the current pause is credited to the new dwell time limit. If the wiper is moving at

the time the dwell time is changed, the new dwell time will be used when the wiper reaches the park

position.

当用户操作间歇档位开关设定更长的间歇时间时，如果雨刮处于停止状态，当前停止时间按照新的间歇

设定时间计时；如果间歇设定时间变化时雨刮处于运动状态，则当雨刮运动至回位位置时，使用新的间歇设

定时间。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

7.13 5.7.13 Hysteresis for Vehicle Speed Class Change/车速档位变化滞回

Requirement Type    Functional

ID    R_Wiper_Wash_5.7.13

Description

The vehicle speed class with hysteresis, a 2km/h hysteresis shall be implemented between each

vehicle speed class. For example, for vehicle speed class 0, if vehicle speed drops below 8 KPH, it enters

V0, and when it exceeds 10 KPH, it is back to V1.

车速档位需要滞回区间，每个车速档位使用 2km/h 作为滞回阈值。例如，车速 0 档在车速降至 8KPH时，进行车速 0 档，当上升至 10KPH 时，进入车速 1 档。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 1, Justified: 0, Failed: 0, Unexecuted: 0, None: 0

8 5.8 Front Wiper Auto Mode/前雨刮自动模式

Requirement Type    Container

ID    R_Wiper_Wash_5.8

Description

Implementation Status

Total: 10, Implemented: 9, Justified: 0, None: 1

Verification Status

Total: 10, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 10

8.1 5.8.1 Auto Mode with RLS Option Enabled/雨量传感器使能自动模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.1

Description

If the EEPROM parameter CFG_RAIN_LIGHT_SENSOR_ENABLED == RLS configured, shall

operate with auto mode when the front wiper intermittent/auto switch is selected.

如果 EEPROM 参数 CFG_RAIN_LIGHT_SENSOR_ENABLED == RLS configured,前雨刮自动/间歇

开关有效时控制前雨刮工作在自动模式。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.2 5.8.2 Front Wiper Control via RLS Request Signal/雨量传感器控制前雨刮

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.2

Description

When the Power mode is in ON and the front wiper auto position is selected, the front wiper is in auto mode (refer to Table 1 Front Wiper Switch Request).

Shall control the front wiper according to the LIN signal RLS_WIPRSPDREQ with park, Low Speed or High Speed mode.

电源模式处于 ON 档且前雨刮自动开关有效，前雨刮工作在自动模式（参考 Table 1 Front Wiper Switch Request）。根据 LIN 信号 RLS_WIPRSPDREQ 控制前雨刮动作：

当 RLS_WIPRSPDREQ == 0x0: park 无雨刮请求，雨刮无动作，

当 RLS_WIPRSPDREQ == 0x5: stage1 刮速1_最低，BDC 控制雨刮低速动作，

当 RLS_WIPRSPDREQ == 0xF: stage7 刮速7_最高，BDC 控制雨刮高速动作。

同时，BDC 根据 RLS 发送的 RLS_WIPRSPDREQ 信号值，直接反馈相应的 RLS_WiprSpdReq 信号值。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.3 5.8.3 Transition between Different Modes/不同模式转移

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.3

Description

The transitions in auto mode from low speed to high speed and from high speed to low speed are

performed only when front wiper reaches park position. Exception is only with splash wipe request

(RLS_SplashReq == 0x1) when wiper is immediately activated in high speed.

When the rain sensor request is changed to off from high speed or low speed mode, shall continue to

activate the wiper at low speed until reach to the park position. If the wiper is already in the park position

when the rain sensor request becomes OFF, shall immediately turn the wiper off.

在自动模式下，前雨刮从低速到高速或从高速到低速的切换是在雨刮运动到回位位置时进行的。不过当

接收到 LIN 信号 RLS_SplashReq == 0x1:

溅水情形触发

时，前雨刮立即切换至高速运动。

当雨量传感器命令请求从高速或低速模式切换至关闭模式，应当继续控制雨刮低速运行至回位位置。如

果前雨刮已经处于回位位置时雨量传感器请求关闭，则立即关闭前雨刮。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.4 5.8.4 Front Wiper Control via RLS Splash Request/雨量传感器溅水命令

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.4

Description

If rain sensor splash request becomes true (RLS_SplashReq == 0x1), the front wiper shall work at high speed mode immediately as long as the splash request is active, which is independent on the signal RLS_WIPRSPDREQ.

When the splash request becomes false (RLS_SplashReq == 0x0), shall activate the front wiper according to signal RLS_WIPRSPDREQ again with park, Low speed or High speed mode.

If signal RLS_WIPRSPDREQ == 0x0;park, shall continue to activate the wiper in low speed until reach to the park position.

如果雨量传感器 RLS_SplashReq ==0x1:

溅水情形触发，前雨刮立即切换至高速运行直至命令不再有效。该命令独立于 RLS_WIPRSPDREQ 信号状态。

如果雨量传感器 RLS_SplashReq == 0x0，切换至 RLS_WIPRSPDREQ 信号状态请求的模式工作。

如果雨量传感器控制命令 RLS_WIPRSPDREQ==0x0; park，则制前雨刮低速运行至回位位置。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.5 5.8.5 RLS Sensitivity Selection/雨量传感器灵敏度调节

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.5

Description

灵敏度值通过 LIN 信号 ICC_BCM_WipeLevel 发送给 RLS。

根据 BDC 发送的灵敏度开关反馈信号 WipeLevelFB 的状态值，以相同的状态值在 LIN4 发送ICC_BCM_WipeLevel

信号。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.6 5.8.6 Increase the RLS Sensitivity by Rheostat Change/档位开关调整灵敏度增加

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.6

Description

When the rain sensor sensitivity is changed to a higher one by Rheostat position change while the

wiper is not moving, an immediate low speed wipe shall be performed.

当雨量传感器灵敏度提高且前雨刮处于停止时，立即启动一次低速刮擦。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.7 5.8.7 Fail Safe Strategy If RLS Fault or Communication Error/故障处理策略

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.7

Description

如果前雨刮处于自动模式，雨量传感器请求低速或高速或 Splash 命令（初始化动作不考虑为低速，初

始动作完成后即认为停止）时， RLS 通信故障，前雨刮应当按照当前模式继续刮擦直至前雨刮开关拨至其

他档位（off，低速或高速）。

如果前雨刮处于 AUTO 档且雨量传感器命令为 OFF（RLS_WIPRSPDREQ==0x0; park）时，发送雨

量传感器故障或 RLS 通信故障，则前雨刮保持停止状态。

已 经 发 生 RLS 通 信 故 障 后 前 雨 刮 开 关 拨 至 AUTO 档 时 ， 如 果 初 始 化 动 作 配 置 使 能 时

(CFG_FRONT_WIPER_INT == Enable)，前雨刮初始化刮擦一次后进入 INT 模式工作。间歇时间参考 5.7.7

Dwell Time with Rheostat and Vehicle Speed/间歇时间。

当故障消失时，前雨刮应该根据雨量传感器信号进行故障。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.8 5.8.8 Change to LOW Mode/自动模式转移至低速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.8

Description

If the front wiper switch is changed to the low speed mode during auto wiping cycle, then the wipers

shall start at low speed mode immediately. If front wiper is in the park position, then it shall be activated

immediately in low speed.

如果前雨刮在自动模式下开关请求变为低速模式，前雨刮立即进入低速模式运行。如果前雨刮处于回位

位置，应当立即启动低速运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.9 5.8.9 Change to HIGH Mode/自动模式转移至高速速模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.9

Description

If the front wiper switch is changed to the high speed mode during auto wiping cycle, then the wipers shall start at high speed mode immediately. If front wiper is in the park position, then it shall be activated immediately in high speed.

如果前雨刮在自动模式下开关请求变为高速模式，前雨刮立即进入高速模式运行。如果前雨刮处于回位位置，应当立即启动高速运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

8.10 5.8.10 洗车模式下禁止自动雨刮功能

Requirement Type    Functional

ID    R_Wiper_Wash_5.8.10

Description

车辆具备洗车模式，由车机端输入洗车模式状态信号，当车辆处于洗车模式或自动洗车模式下时，如果此时雨刮开关处于 aotu 档，则 BDC 不响应 RLS 发送的自动雨刮动作信号 RLS_WIPRSPDREQ。

电源模式为 ON 时：

1） 当接收到 PDCU_CarWashModSts == 0x1：洗车模式 or 0x2：自动洗车模式

，BDC 禁止自动雨刮使能。

2） 当接收到 PDCU_CarWashModSts == 0x0：初始值，BDC 不再禁止自动雨刮使能。

当雨刮不再停止位时，BDC 接收到 PDCU_CarWashModSts ==0x1或

0x2，此时控制雨刮回到停止位后，再响应洗车模式下禁止自动雨刮功能。

洗车模式下不禁止用户操作雨刮开关进入雨刮低速模式或雨刮高速模式。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

9 5.9 Front Wiper Mist Mode/前雨刮 Mist 操作

Requirement Type    Functional

ID    R_Wiper_Wash_5.9

Description

The inputs of front wiper switches for Mist mode is same as the low speed mode, Will control the wiper

working in the same way as low speed mode.

前雨刮开关 MIST 输入信号与低速操作相同，按照低速模式开关控制前雨刮动作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10 5.10 Front Washer and Wash Wiping /前洗涤及雨刮洗涤联动

Requirement Type    Container

ID    R_Wiper_Wash_5.10

Description

Implementation Status

Total: 13, Implemented: 13, Justified: 0, None: 0

Verification Status

Total: 13, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 13

10.1 5.10.1 Activation of Front Washer/前洗涤开启

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.1

Description

When the front washer switch is ON, and Power mode is ON , shall activate the front washer pump to wash the front windscreen immediately by set WP_Front_Washer_CMD to ON. The front washer shall remain on while the front washer switch is pressed and the time PAR_FW_WASHER_MAX_ACTIVATION is not expired.

当 前 洗 涤 开 关 处 于 ON 且 电 源 模 式 处 于 ON 时 ， 立 即 开 启 前 洗 涤 电 机 控 制 输 出WP_Front_Washer_CMD 清洗风窗。前洗涤开关一直按下则前洗涤电机持续工作，直至开关按下时间超过PAR_FW_WASHER_MAX_ACTIVATION。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.2 5.10.2 Deactivation of Front Washer/关闭前洗涤

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.2

Description

If the front washer switch is released or Power mode is switched to OFF or ACC or CRANK during washer activating or the maximum activation time PAR_FW_WASHER_MAX_ACTIVATION is reached,

the washer shall be deactivated immediately.

如 果 前 洗 涤 开 关 松 开 或 电 源 模 式 拨 至 OFF ， 或 开 关 操 作 时 间 超 过PAR_FW_WASHER_MAX_ACTIVATION 时，立即关闭前洗涤。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.3 5.10.3 Not Activation of Front Wash Wiping/前雨刮洗涤联动不开启

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.3

Description

If the front washer switch is pressed less than PAR_FW_WASH_WIPING_DELAY, shall not activate

the wash wiping function.

如果前洗涤开关按下时间小于PAR_FW_WASH_WIPING_DELAY 时，不激活雨刮洗涤联动功能。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.4 5.10.4 Activation of Front Wash Wiping during INT Mode or OFF Mode/OFF 或 INT 模 式激活雨刮洗涤联动

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.4

Description

If the front washer switch is activated more than PAR_FW_WASH_WIPING_DELAY, shall activate

the front wiper low speed wiping with a delay of PAR_FW_WIPING_DELAY_TIME from washer switch activation, until the washer function is deactivated. And at the end of washing activation, the wiper shall

continue to do a prolongation wiping.

如 果 前 洗 涤 开 关 激 活 时 间 超 过 PAR_FW_WASH_WIPING_DELAY ， 则 在 前 洗 涤 开 关 开 启PAR_FW_WIPING_DELAY_TIME 延时后开启前雨刮低速运行直至前洗涤功能关闭。前洗涤功能关闭后，

前雨刮应继续刮擦一定次数。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.5 5.10.5 Activation of Front Wash Wiping during LOW or HIGH Mode/LOW 或 HIGH 模 式激活雨刮洗涤联动

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.5

Description

If the current front wiper is in low speed or high speed mode, then wiper shall continue to wipe with

respectively mode, and no prolongation wipe shall be followed.

当前雨刮处于低速或高速模式时，前雨刮继续按照相应的模式工作，没有前雨刮延续动作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.6 5.10.6 Activation of Front Washer during Auto Mode with RLS Request is Low Speed  or High Speed Mode or Splash Mode/自动模式下 RLS 请求为低速或高速或 Splash 时 激活雨刮洗涤联动

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.6

Description

If the front washer switch is activated when front wiper is in auto mode with rain sensor request is low

speed or high speed or splash, shall continue the current wiper mode during the washing time.

The front wiper mode shall be frozen during the washing time and ignore the new requests from rain

sensor, which will change the wiper mode from high speed to low speed/off OR from low speed to off.

The washing process shall include the prolongation wiping.

After prolongation wiping is finished, shall activate the front wiper again according to the rain sensor

request.

如果在前雨刮处于 AUTO 模式下，雨量传感器请求为低速或高速或 Splash 命令时，前洗涤开关按下，

应继续当前的雨刮工作模式。

在清洗过程中，保持前雨刮的工作模式并忽略所有雨量传感器请求从高速到低速/OFF 或低速到 OFF

的命令。

洗涤过程应包括延续刮擦功能。当延续刮擦完成后，按照雨量传感器的请求控制前雨刮工作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.7 5.10.7 Activation of Front Wash Wiping during Auto Mode with RLS Request is OFF/ 自动模式下 RLS 请求为 OFF 时激活雨刮洗涤联动

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.7

Description

If the front washer switch is activated more than PAR_FW_WASH_WIPING_DELAY when front wiper

is in auto mode with rain sensor request is OFF, shall activate the front wiper low speed wiping with a

delay of PAR_FW _WIPING_DELAY_TIME from washer activation, until the washer function is deactivated.

The front wiper mode shall be frozen during the washing time and ignore the new requests from rain

sensor, which will change the wiper mode from off to low / high speed mode or from off to splash mode.

The washing process shall include the prolongation wiping.

After prolongation wiping is finished, shall activate the front wiper again according to the rain sensor

request.

前 雨 刮 处 于 自 动 模 式 且 雨 量 传 感 器 请 求 为 OFF 时 ， 前 洗 涤 开 关 有 效 时 间 超 过

PAR_FW_WASH_WIPING_DELAY 时间，在前洗涤开关开启时间超过 PAR_FW_ WIPING_DELAY_TIME

后控制前雨刮低速运行，直至前洗涤关闭。

在清洗过程中，保持前雨刮的工作模式并忽略所有雨量传感器请求从 OFF 到低速/高速模式或从 OFF 到

Splash 模式的命令。

洗涤过程应包括延续刮擦功能。当延续刮擦完成后，按照雨量传感器的请求控制前雨刮工作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.8 5.10.8 Front Washer Prolongation Wiping/雨刮联动延续刮擦

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.8

Description

At the end of washing activation, the front wiper shall continue to wipe for a further

PAR_FW_PROLON_WIPE_CYCLE wipers at low speed and then waiting

PAR_FW_DRIP_WIPE_WAIT_TIME do a drip wipe further. If parameter

PAR_FW_PROLON_WIPE_CYCLE is set to 0, then no prolongation wipe will be executed.

当前洗涤功能结束时，前雨刮应以低速模式继续刮擦 PAR_FW_PROLON_WIPE_CYCLE 次后停顿

PAR_FW_DRIP_WIPE_WAIT_TIME 再多刮一次。如果参数 PAR_FW_PROLON_WIPE_CYCLE 设置为

0，则雨刮联动功能不执行延续刮擦。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.9 5.10.9 Change the Front Wiper Switch during Wash Wiping or Prolongation Wiping/ 洗涤联动或联动延续刮擦过程中前雨刮开关变化

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.9

Description

If front wiper switch is changed to low speed or high speed during wash or prolongation wiping, the

prolongation wiping should be cancelled, and start new wiper mode immediately.

If front wiper switch is changed to intermittent mode during wash or prolongation wiping, the wiper

shall finish the prolongation wiping at low speed before adopting the new intermittent mode, and after that

the dwell time shall be followed.

If front wiper switch is changed to auto mode during wash or prolongation wiping, the requests from

rain sensor, which will change the wiper mode from high speed to low speed/off OR from low speed to off,

shall be ignored until the prolongation wiping is finished.

If front wiper switch is changed to OFF during wash or prolongation wiping, the wiper shall finish the prolongation wiping at low speed.

如果在洗涤联动或延续刮擦过程中，前雨刮开关变为低速或高速模式，则延续刮擦功能取消，立即执行前雨刮的新模式请求。

如果洗涤联动或延续刮擦过程中，前雨刮开关变为 INT 模式，前雨刮应低速运行完成延续刮擦后按照新的前雨刮间歇模式运行。

如果洗涤联动或延续刮擦过程中，前雨刮开关变为 AUTO 模式，且应当忽略雨量传感器的请求从高速到低速/OFF 或低速到 OFF 的命令，并完成延续刮擦。

如果洗涤联动或延续刮擦过程中，前雨刮开关变为 OFF，前雨刮应低速运行完成延续刮擦功能。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.10 5.10.10 Ignition Changed to OFF during Wash Wiping/雨刮洗涤联动过程电源模式变为 OFF

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.10

Description

If Power mode is changed to OFF during wash or prolongation wiping , the wiper shall be stopped

immediately. The left wipe cycles will be cancelled.

如果在洗涤联动或延续刮擦过程中电源模式变为 OFF 时，前雨刮立即停止工作。剩余的刮擦次数也取消。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.11 5.10.11 Detection of a Faulty Front Washer Switch/前洗涤开关故障检测

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.11

Description

If the front washer switch is detected continually active more than

PAR_FW_WASHER_SW_STUCK_TIME and Power mode is in ON, its value shall be ignored and a DTC

of front washer switch failure shall be set.

如果前洗涤开关有效时间超过 PAR_FW_WASHER_SW_STUCK_TIME 且电源模式处于 ON 档，则

忽略前洗涤开关状态，并记录前洗涤开关故障 DTC。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.12 5.10.12 Behavior after a Faulty Front Washer Switch/前洗涤开关故障处理

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.12

Description

After detection of a faulty front washer switch, if the front washer switch is detected as inactive again

when Power mode is ON, then the new valid request of washer switch will be reacted afterwards and the

corresponding DTC shall be cleared.

检测到前洗涤开关故障后，在电源模式处于 ON 档时前洗涤开关会一直被认为无效状态，直至新的前

洗涤开关再次被操作时，清除相应的 DTC。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

10.13 5.10.13 Feedback of Front Washer Output Status/前洗涤输出状态反馈

Requirement Type    Functional

ID    R_Wiper_Wash_5.10.13

Description

将前洗涤状态反馈到 CAN 总线上。

当前洗涤输出时，设置 CAN 信号 FrntWshrCmd= ON

当前洗涤关闭时，设置 CAN 信号 FrntWshrCmd= OFF

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

11 5.11 Front Wiper Park Position/前雨刮回位位置

Requirement Type    Container

ID    R_Wiper_Wash_5.11

Description

Implementation Status

Total: 2, Implemented: 1, Justified: 1, None: 0

Verification Status

Total: 2, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 1

11.1 5.11.1 Reaction Time to the Detection of Park Switch Close/回位信号响应时间

Requirement Type    Functional

ID    R_Wiper_Wash_5.11.1

Description

When the park switch closes, the wiper relays shall be switched OFF. The tolerance of the time delay that can be accepted between the park switch closing (after signal debounce finished) and the relay turning

OFF is PAR_FW_PARK_RES_TIME msec. (This effects the park position of the wiper arms on the windscreen.)

当回位开关闭合，前雨刮继电器应当切换至 OFF 状态。回位开关闭合到继电器关闭之间的时间要求为PAR_FW_PARK_RES_TIME（这会影响雨刮臂在风挡上停止位置）。

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

11.2 5.11.2 Back to Park Position When Ignition switched ON/电源模式拨至 ON 时回位

Requirement Type    Functional

ID    R_Wiper_Wash_5.11.2

Description

Following a transition Power mode from OFF or ACC or CRANK to ON with the front wiper column stalk in the OFF position and the front wiper park switch in the open position, shall activate the front wiper relays to allow the wipers to run at low speed until reach the park position.

当电源模式从 OFF 拨至 ON 档且雨刮手柄处于 OFF 位置，如果前雨刮不处于回位位置，应控制前雨刮低速运行至回位位置。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

12 5.12 Front Wiper Stall Protection/前雨刮堵转保护

Requirement Type    Container

ID    R_Wiper_Wash_5.12

Description

Implementation Status

Total: 2, Implemented: 2, Justified: 0, None: 0

Verification Status

Total: 2, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 2

12.1 5.12.1 Deactivate Front Wiper after Maximum Wait Time for Wiper Reaching Park  Position/最大等待雨刮回位时间

Requirement Type    Functional

ID    R_Wiper_Wash_5.12.1

Description

The BCM shall use the signal WP_Front_Wiper_Park_Sts_SW from wiper park switch to detect the stalling of the front wiper mechanism. When the front wiper is enabled, the wiper park switch shall be monitored.

If the park switch status does not change during a period of PAR_FW_WAIT_STALL_TIME，then the front wiper relays shall be deactivated and a DTC of front wiper park switch failure shall be set .

Note: Stall protection shall be active during ALL wiper functions.

当前雨刮工作时，检测前雨刮回位信号来判断前雨刮机构是否堵转。

如果回位信号在 PAR_FW_WAIT_STALL_TIME 时间内没有变化，则关闭前雨刮继电器输出，并记录相应的 DTC。

注：所有雨刮功能执行时都进行堵转检测和保护。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

12.2 5.12.2 Reactivate Front Wiper after Stall Protection/前雨刮堵转保护后再次开启

Requirement Type    Functional

ID    R_Wiper_Wash_5.12.2

Description

The wiper motor can't be restarted until any of the following conditions is met:

2） Wash wiping is activated again

3） Power mode turned to OFF or ACC or CRANK and to ON again.

直到任一如下条件满足时，前雨刮电机可以再次开启：

2） 前雨刮洗涤联动功能激活

3） 电源模式从 OFF 拨至 ON

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13 5.13 Front Wiper Maintenance Mode/前雨刮维修模式

Requirement Type    Container

ID    R_Wiper_Wash_5.13

Description

隐藏式前雨刮在正常停止后无法抬起刮臂，为方便日常雨刮片的维修保养和洗车，设计前雨刮维修模式。当 CFG_FW_MAINTENANCE_MODE_ENABLED== enable 时，允许进入雨刮维修模式。

Implementation Status

Total: 10, Implemented: 9, Justified: 1, None: 0

Verification Status

Total: 10, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 9

13.1 5.13.1 网络输入进入前雨刮维修模式

Requirement Type    Container

ID    R_Wiper_Wash_5.13.1

Description

Implementation Status

Total: 3, Implemented: 3, Justified: 0, None: 0

Verification Status

Total: 3, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 3

13.1.1 5.13.1.1 开启前雨刮维修模式条件

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.1.1

Description

当满足以下所有条件时，收到车机信号 WiprMntnModSwt==ON，前雨刮进入维修模式，并以低

速运行至风窗玻璃左侧位置后停止。

1) 电源模式处于 ON 状态；

2) 前雨刮处于复位位置（复位信号有效）；

3) 前舱盖处于关闭状态；

注：进入前雨刮维修模式后，禁用正常前雨刮功能。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.1.2 5.13.1.2 Action of Front Wiper Maintenance Mode/维修模式下前雨刮动作

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.1.2

Description

当 满 足 前 雨 刮 维 修 模 式 条 件 后 ， 前 雨 刮 以 低 速 动 作

PAR_FW_MAINTENANCE_MODE_RUNNING_TIMER sec 时间，使前雨刮刮片运行至前风窗玻璃左

侧位置后停止，并发送网络信号前雨刮处于维修模式 FrntWiprMntnSts == MAINTENANCE Mode。

当雨刮处于处于维修模式状态时，不相应雨量光及雨刮开关动作请求。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.1.3 5.13.1.3 Quit The Front Wiper Maintenance Mode/退出前雨刮维修模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.1.3

Description

当 满 足 以 下 所 有 条 件 时 ， 前 雨 刮 以 低 速 运 行 至 复 位 区 后 退 出 前 雨 刮 维 修 模 式 发 送FrntWiprMntnSts== NOMAL_STATE。

1) 前雨刮处于维修模式；

2) 电源模式为 ON；

3) 收到车机信号 WiprMntnModSwt==OFF；

注：当前雨刮在维修模式下由风窗最左侧回到复位位置的过程中，若发生雨刮堵转，则退出前雨刮

维修模式。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.2 5.13.2 开关输入进入前雨刮维修模式

Requirement Type    Container

ID    R_Wiper_Wash_5.13.2

Description

当 CFG_FW_MAINTENANCE_MODE_ENABLED== enable 时，接收到雨刮开关输入的雨刮维

修请求，执行雨刮维修模式

Implementation Status

Total: 4, Implemented: 4, Justified: 0, None: 0

Verification Status

Total: 4, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 4

13.2.1 5.13.2.1 开启前雨刮维修模式条件

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.2.1

Description

当满足以下所有条件时，操作前雨刮开关 MIST 档有效，前雨刮进入维修模式，并以低速运行至风

窗玻璃左侧位置后停止。

1) 前雨刮处于复位位置（复位信号有效）；

2) 前舱盖处于关闭状态；

3) 电源模式由 ON 变为 OFF 档后的

PAR_FW_MAINTENANCE_MODE_ACTIVE_DELAY_TIMER 时间内；

注：进入前雨刮维修模式后，禁用正常前雨刮功能。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.2.2 5.13.2.2 定时器关闭

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.2.2

Description

当满足以下任一条件时，关闭 PAR_FW_MAINTENANCE_MODE_ACTIVE_DELAY_TIMER 定

时器。

1）定时器 PAR_FW_MAINTENANCE_MODE_ACTIVE_DELAY_TIMER 超时；

2）前雨刮 MIST 档位有效，进入前雨刮维修模式；

3) 电源模式由 OFF 档位，变为 ON 档位。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.2.3 5.13.2.3 Action of Front Wiper Maintenance Mode/维修模式下前雨刮动作

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.2.3

Description

1、前雨刮运行至风窗玻璃左侧位置

当 满 足 前 雨 刮 维 修 模 式 条 件 后 ， 前 雨 刮 以 低 速 动 作PAR_FW_MAINTENANCE_MODE_RUNNING_TIMER sec 时间，使前雨刮刮片运行至前风窗玻璃左

侧位置后停止，并发送网络信号前雨刮处于维修模式 FrntWiprMntnSts== MAINTENANCE Mode

。

。

2、前雨刮运行至复位位置

当前雨刮处于维修状态时，将电源模式由 OFF 变为 ON 档后，满足以下任一条件后，前雨刮以低

速运行至复位位置后停止。

1) 前雨刮开关状态变化；

备注：前雨刮开关状态变化指，前雨刮 MIST 档、低速档、高速档、间歇档开关状态变化。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.2.4 5.13.2.4 Quit The Front Wiper Maintenance Mode/退出前雨刮维修模式

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.2.4

Description

当前雨刮在维修模式下运行至复位位置后，满足以下任一条件时，退出前雨刮维修模式发送

FrntWiprMntnSts == Normal Mode，并按正常逻辑处理。

1) 点火开关为 ON 变为 OFF；

2) 前雨刮位于雨刮维修模式时，检测到前雨刮开关网络信号有效(高/低/自动成 INT)，执行雨刮

动作后，前雨创处于复位位置《复位信号有效)；

3) 前洗涤开关状态变化；

注：当前雨刮在维修模式下由风窗最左侧回到复位位置的过程中，若发生雨刮堵转，则退出前雨刮

维修模式。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.3 5.13.3 Open The Hood When Under The Front Wiper Maintenance Mode/维修模式下 前舱盖打开

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.3

Description

前雨刮维修模式下雨刮运行中，前舱盖打开，禁止前雨刮动作。

当关闭前舱盖后，维修模式下的雨刮动作继续运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

13.4 5.13.4 前雨刮维修模式存储

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.4

Description

前雨刮维修模式态应当存在在非易失性（NVM）存储器，当 ECU 复位后，前雨刮维修模式状态应当恢复至复位前存储在 NVM 中的状态。

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

13.5 5.13.5 前雨刮维修模式下电源模式变为 OFF 后雨刮动作

Requirement Type    Functional

ID    R_Wiper_Wash_5.13.5

Description

当 CFG_FW_MAINTENANCE_MODE_ENABLED== enable 时，电源模式由 ON 跳至 OFF 时，前雨刮动作不立即停止，完成本次挂刷周期后在回位区停止。

当 CFG_FW_MAINTENANCE_MODE_ENABLED== disable 时，按照第本章前十节要求，电源模

式由 ON 跳至 OFF 时，立即停止雨刮动作。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

14 5.14 Rear Wiper Switch Request/后雨刮开关请求（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.14

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

15 5.15 Rear Washer and Wash Wiping/后洗涤及雨刮洗涤联动（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.15

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

16 5.16 Reverse Gear Wiping/倒车雨刮联动（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.16

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

17 5.17 Rear Wiper and Washer Inhibit/后雨刮及洗涤禁止（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.17

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

18 5.18 Rear Wiper Park Position/后雨刮回位位置（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.18

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

19 5.19 Rear Wiper Stall Protection/后雨刮堵转保护（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.19

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

20 5.20 Voice Control of Front Wiper in Low Speed Mode/语音控制前雨刮低速运行

Requirement Type    Container

ID    R_Wiper_Wash_5.20

Description

此项功能可以通过 EEPROM 配置参数使能。当 Cfg_Wiper_Smart_Voice_Control ==Enabled，接收车机发送的 CAN 网络信号，控制前雨刮低速运行。

Implementation Status

Total: 8, Implemented: 2, Justified: 0, None: 6

Verification Status

Total: 8, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 8

20.1 5.20.1 Activation of Front Washer Voice Control/前雨刮语音控制功能开启

Requirement Type    Container

ID    R_Wiper_Wash_5.20.1

Description

Implementation Status

Total: 2, Implemented: 2, Justified: 0, None: 0

Verification Status

Total: 2, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 2

20.1.1 5.20.1.5 Activation of Front Wiper Voice Control/前雨刮语音控制功能开启条件

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.1.5

Description

以下所有条件满足时，开启前雨刮语音控制功能。

1) 电源模式在 ON 档；

2) 前洗涤处于 OFF 档；

3) 前雨刮开关处于 OFF 档；

4) 收到车机发送的 CAN 信号前雨刮请求

（FrntWiprReq==ON）时；

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

20.1.2 5.20.1.6 Output When Activating Front Wiper Voice Control/前雨刮语音控制功能开启动作

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.1.6

Description

当前雨刮语音控制功能开启，控制前雨刮低速运行。

Implementation Status

Total: 1, Implemented: 1, Justified: 0, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

20.2 5.20.2 Deactivation of Front Wiper Voice Control/前雨刮语音控制功能退出

Requirement Type    Container

ID    R_Wiper_Wash_5.20.2

Description

Implementation Status

Total: 5, Implemented: 0, Justified: 0, None: 5

Verification Status

Total: 5, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 5

20.2.1 5.20.2.1 Deactivation of Front Wiper Voice Control/前雨刮语音控制退出条件

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.2.1

Description

当满足以下任一条件时，退出前雨刮语音控制。

1) 电源模式由 ON 档位变为 OFF；

2) 收到前洗涤开关有效；

3) 前雨刮开关由 OFF 拨到非 OFF 档位。

4) 收到车机发送的 CAN 信号关闭前雨刮请求（FrntWiprReq==OFF）时

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

20.2.2 5.20.2.2 Deactivation of Front Wiper Voice Control In the case of iginition status change /当电源模 式变化时退出前雨刮语音控制

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.2.2

Description

当因电源模式变化退出前雨刮语音控制时，按照 5.1.1 节错误!未找到引用源。中规定的电源模式进行

处理。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

20.2.3 5.20.2.3   Deactivation of Front Wiper Voice Control In the Case of Front Washer Switch Change 前 洗涤开关有效退出前雨刮语音控制

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.2.3

Description

因前洗涤开关有效，退出前雨刮语音控制时，按照洗涤开关状态输出。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

20.2.4 5.20.2.4   Deactivation of Front Wiper Voice Control In the Case of Front Wiper Switch Change    /前 雨刮开关由 OFF 拨到非 OFF 档位退出前雨刮控制。

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.2.4

Description

因前雨刮开关由 OFF 拨到非 OFF 档位，退出前雨刮语音控制时，按照雨刮开关状态输出。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

20.2.5 5.20.2.5 Deactivation  of  Front  Wiper  Voice  Control  In  the  Case  of  Receiving  Voice  Control  of  Deactivating Front Wiper/收到语音控制命令退出前雨刮语音控制

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.2.5

Description

因收到收到车机发送的 CAN 信号关闭前雨刮请求（FrntWiprReq==OFF）时，根据 4.1 节中规定处理。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

20.3 5.20.3 Conflicts Management during of Voice Control Front Wiper/冲突处理

Requirement Type    Functional

ID    R_Wiper_Wash_5.20.3

Description

在语音控制前雨刮输出过程中，收到开启语音控制前雨刮打开请求（FrntWiprReq==ON

，

维持前雨刮，

维持前雨刮输出状态。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21 5.21 Voice Control of Front Washer/前洗涤及雨刮洗涤联动语音控制

Requirement Type    Functional

ID    R_Wiper_Wash_5.21

Description

此项功能可以通过配置项使能。当 CFG_Frontwash_Smart_Voice_Control ==Enabled

，

接收车机发

送的 CAN 网络信号，控制前雨刮洗涤并进行洗涤联动及延续刮擦。

注：若在语音控制前洗涤及洗涤联动、延续刮擦的过程中，配置项变化对其无影响，按照本节规范输出。

退出本次语音洗涤后，再响应新的配置项。

Implementation Status

Total: 14, Implemented: 0, Justified: 0, None: 14

Verification Status

Total: 14, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 14

21.1 5.21.1 Activation of Front Washer Voice Control/前洗涤语音控制功能开启

Requirement Type    Functional

ID    R_Wiper_Wash_5.21.1

Description

Implementation Status

Total: 5, Implemented: 0, Justified: 0, None: 5

Verification Status

Total: 5, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 5

21.1.1 5.21.1.1 Activation of Front Washer Voice Control/前洗涤语音控制功能开启条件

Requirement Type    Functional

ID    #111

Description

以下所有条件满足时，开启前洗涤语音控制功能。

1) 电源模式在 ON 档；

2) 前洗涤处于 OFF 档；

3) 前雨刮开关处于 OFF 档；

4) 收到车机发送的 CAN 信号前洗涤请求

（

FrntWshrWiprReq==ON

）

时；

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.1.2 5.21.1.2 Front Wash Output when Activating Front Washer Voice Control /前洗涤语音控制开启动作

Requirement Type    Functional

ID    #112

Description

当前洗涤语音控制开启时，控制前洗涤输出时间 PAR_FW_WASHER_SMART_VOICE_ACTIVATION

s；

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.1.3 5.21.1.3 Front Wash Wiping when Activating Front Washer Voice Control /前洗涤语音控制雨刮联动

Requirement Type    Functional

ID    #113

Description

在语音控制前洗涤输出 PAR_FW_WASHER_SMART_VOICE _DELAY_TIME

（

1s

）

时间后，开启前

雨刮低速运行直至前洗涤 PAR_FW_WASHER_SMART_VOICE_ACTIVATION 结束。前洗涤智能刮洗结束

后，前雨刮应继续执行延续刮擦动作。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.1.4 5.21.1.4 Prolongation Wiping when Activating Front Washer Voice Control /前洗涤语音控制延续刮擦

Requirement Type    Functional

ID    #114

Description

智能语音控制前洗涤功能结束时，前雨刮应以低速模式继续刮擦 PAR_FW_PROLON_WIPE_CYCLE

次 后 停 顿 PAR_FW_DRIP_WIPE_WAIT_TIME msec 再 多 刮 一 次 。 如 果 参 数

PAR_FW_PROLON_WIPE_CYCLE 设置为 0，则雨刮联动功能不执行延续刮擦。

注：当语音控制前雨刮运行时，收到车机的语音洗涤命令（FrntWshrWiprReq==ON），

前雨刮继续低速运行，没有前雨刮延续刮擦动作。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.2 5.21.2 Deactivation of Front Washer Voice Control/前洗涤语音控制退出

Requirement Type    Functional

ID    R_Wiper_Wash_5.21.2

Description

Implementation Status

Total: 6, Implemented: 0, Justified: 0, None: 6

Verification Status

Total: 6, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 6

21.2.1 5.21.2.5 Deactivation of Front Washer Voice Control/前洗涤语音控制退出条件

Requirement Type    Functional

ID    #116

Description

当满足以下任一条件时，退出前洗涤语音控制。语音控制前洗涤不再输出，语音洗涤联动不再执行。

1) 电源模式由 ON 档位变为 OFF。

2) 收到前洗涤开关有效

3) 前雨刮开关由 OFF 拨到非 OFF 档位。

4) 收到车机发送的 CAN 信号前雨刮请求（FrntWshrWiprReq ==ON）时

注：当定时器超时，清除定时器。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.2.2 5.21.2.6 Deactivation of Front Washer Voice Control In the case of iginition status change /当电源 模式变化时退出前洗涤语音控制

Requirement Type    Functional

ID    #117

Description

电源模式变化退出前洗涤语音控制时，按照 4.1 节中规定的电源模式进行处理。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.2.3 5.21.2.7   Deactivation of  Front Washer Voice Control  In  the Case of Front Wash Switch Status  Change 前洗涤开关有效退出前洗涤语音控制

Requirement Type    Functional

ID    #118

Description

前洗涤开关有效，退出前洗涤语音控制时，语音控制前洗涤输出、联动和延续刮擦停止。按照 4.8 节进

行处理。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.2.4 5.21.2.8   Deactivation of Front Washer Voice Control In the Case of Front Wash Status Change/前 雨刮开关由 OFF 拨到非 OFF 档位退出前洗涤语音控制。

Requirement Type    Functional

ID    #119

Description

前雨刮开关由 OFF 拨到非 OFF 档位，退出前洗涤语音控制时，前洗涤输出停止，联动和延续刮擦按

照《洗涤联动或联动延续刮擦过程中前雨刮开关变化》处理；

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.2.5 5.21.2.9 Deactivation of Front Washer Voice Control In the Case of Receiving Voice Control of Front  Wiper/收到前雨刮语音控制命令退出前洗涤语音控制

Requirement Type    Functional

ID    #120

Description

收到前雨刮语音控制命令退出前洗涤语音控制时，语音控制前洗涤输出、联动和延续刮擦停止，根据

5.20.1.6 操作。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.3 5.21.3 Conflicts Management during of Voice Control Front Wash/冲突处理

Requirement Type    Functional

ID    R_Wiper_Wash_5.21.3

Description

在语音洗涤、雨刮联动和延续刮擦的过程中，收到新的语音洗涤前风窗命令，不再响应新的清洗前风窗

请求。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

21.4 5.21.4 Fault Management/故障处理

Requirement Type    Functional

ID    R_Wiper_Wash_5.21.4

Description

连续 PAR_FW_WASHER_SW_STUCK_TIMEs 持续收到 AVN 发送的 FrntWshrWiprReq==ON，判断节点故障，并记录 DTC。此时不再执行语音洗涤功能，直至 DTC 被清除。

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

22 5.22 Front Wiper at Auto Drive Mode/自动驾驶模式下的自动前雨刮功能（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.22

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

23 5.23 雨刮开关信号发送（预留）

Requirement Type    Functional

ID    R_Wiper_Wash_5.23

Description

Implementation Status

Total: 1, Implemented: 0, Justified: 1, None: 0

Verification Status

Total: 1, Passed: 0, Justified: 1, Failed: 0, Unexecuted: 0, None: 0

24 5.24 雨量光需求固定信号发送说明

Requirement Type    Functional

ID    R_Wiper_Wash_5.24

Description

根据 RLS 信号需求，本项目固定发送以下 LIN 信号：

BCM_WipeSysType == 0x0: 2-stage wiper All 2

速雨刮系统

BCM_LightThdCode == 0x7: Sensitivity 7/

灯光阈值档位7 4600/6000Lux BDC_WashSt周期发送洗涤状态实时状态， BDC_FrntWiprParkSt周期发送雨刮回位状态，

BDC_550nmTrsm、BDC_800nmTrsm发0x7F，

BCM_VehTypeCode周 期 发 车 型 代 码 ；BCM_WipeSysType周期发

0x0；备注：BCM_VehTypeCode 根据项目配置

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

25 5.25 雨量光需求检测信号发送说明

Requirement Type    Functional

ID    R_Wiper_Wash_5.25

Description

根据 BDC 自身采集的输入信号，根据采集结果发送以下信号：

BCM_PwrMode（电源模式）、BDC_FrntWiprParkSt（雨刮位置）

Implementation Status

Total: 1, Implemented: 0, Justified: 0, None: 1

Verification Status

Total: 1, Passed: 0, Justified: 0, Failed: 0, Unexecuted: 0, None: 1

26

Requirement Type    N/A

ID    #7

Description

26.1 no need to do debounce by the model

Requirement Type    N/A

ID    J_Wiper_Wash_debounce

Description

26.2 input and output signal description only

Requirement Type    N/A

ID    J_Wiper_Wash_signal_description

Description

26.3 Reserved

Requirement Type    N/A

ID    J_Wiper_Wash_Reserved

Description

26.4 no software related design

Requirement Type    N/A

ID    J_Wiper_Wash_Not_Model_Software

Description